LAB 5: CNN AND DE10-Nano INTEGRATION

# Contents

1.	Contents	2

2.	Introduction	3

3.	Training your CNN Model	3

4.	Setting up your FPGA CNN	6

conv_full.v	6

maxpool.v	9

fc_layer.v	10

argmax.v	12

cnn_core.v	13

5.	Connecting the D8M to the FPGA	14

6.	CNN Inference through HPS BareMetal	14

# Introduction

In this lab we will look into the implementation of a handwritten-digit recognition system on the DE10-Nano Board. We will use MNIST dataset to train CNN which we will then use on the FPGA along with D8M-GPIO camera.

It is possible to use a similar structure for the mini-project.

# Training your CNN Model

This part trains the following CNN and converts it into fixed-point weight files ( .hex ) that the Verilog design reads.

We will use Python and Pytorch to train the CNN. Open up a Jupyter notebook:

import torch

print("CUDA Available:", torch.cuda.is_available())

device = torch.device("cuda" if torch.cuda.is_available() else "cpu")

import torchvision

import torchvision.transforms as transforms

transform = transforms.Compose([transforms.ToTensor()])

train_dataset = torchvision.datasets.MNIST(root='./data', train=True, download=True, transform=transform)

test_dataset = torchvision.datasets.MNIST(root='./data', train=False, download=True, transform=transform)

import torch.nn as nn

class SimpleCNN(nn.Module):

def __init__(self):

super(SimpleCNN, self).__init__()

self.conv1 = nn.Conv2d(in_channels=1, out_channels=4, kernel_size=3, padding=0)

self.relu = nn.ReLU()

self.pool = nn.MaxPool2d(kernel_size=2, stride=2)

self.fc = nn.Linear(in_features=676, out_features=10)

def forward(self, x):

x = self.conv1(x) # -> [batch, 4, 26, 26]

x = self.relu(x)

x = self.pool(x) # -> [batch, 4, 13, 13]

x = x.view(x.size(0), -1) # flatten -> [batch, 676]

x = self.fc(x) # -> [batch, 10]

return x

import torch.optim as optim

from torch.utils.data import DataLoader

model = SimpleCNN().to(device)

train_loader = DataLoader(train_dataset, batch_size=64, shuffle=True)

test_loader = DataLoader(test_dataset, batch_size=64, shuffle=False)

criterion = nn.CrossEntropyLoss()

optimizer = optim.Adam(model.parameters(), lr=0.001)

for epoch in range(10):

model.train()

running_loss = 0.0

for images, labels in train_loader:

images, labels = images.to(device), labels.to(device)

optimizer.zero_grad()

outputs = model(images)

loss = criterion(outputs, labels)

loss.backward()

optimizer.step()

running_loss += loss.item()

print(f"Epoch loss: {running_loss/len(train_loader):.4f}")

torch.save(model.state_dict(), 'simple_cnn_mnist.pth')

Now you have a trained CNN model. It may be necessary to tweak the design to fit your requirements for accuracy.

Next quantize your weights such that you can feed this into the FPGA through Quartus.

import numpy as np

SCALE = 4096

def quantize(x, scale=SCALE):

return np.round(x * scale).astype(np.int64)

sd = model.state_dict()

conv_w_q = quantize(sd['conv1.weight'].cpu().numpy()) # [4,1,3,3]

conv_b_q = quantize(sd['conv1.bias'].cpu().numpy()) # [4]

fc_w_q = quantize(sd['fc.weight'].cpu().numpy()) # [10,676]

fc_b_q = quantize(sd['fc.bias'].cpu().numpy()) # [10]

Once the weights have been quantized export them in the form of .hex files

def to_hex16(val):

val = int(val)

if val < 0:

val = (1 << 16) + val

return f"{val & 0xFFFF:04x}"

def save_hex(filename, values_flat):

with open(filename, 'w') as f:

for v in values_flat:

f.write(to_hex16(v) + "\n")

save_hex('conv_weight.hex', conv_w_q.flatten()) # 36 lines (4 filters × 9)

save_hex('conv_bias.hex', conv_b_q.flatten()) # 4 lines

save_hex('fc_weight.hex', fc_w_q.flatten()) # 6760 lines (10×676)

save_hex('fc_bias.hex', fc_b_q.flatten()) # 10 lines

These four .hex files must be copied into the Quartus project folder (same folder as the .v source files) before compiling. The Verilog design loads them at synthesis time using $readmemh .

# Setting up your FPGA CNN

You will receive the FPGA files for the example project which has already implemented the CNN and the D8M. Let us go through each of the Verilog files and how they work.

For each layer of the CNN we will construct a separate Verilog file.

## conv_full.v

conv_full.v  holds the convolution layer, ReLU and Rescale layers. The Verilog file is adjusted to read the camera image from the write port.

This module performs a 3×3 convolution with 4 output filters over a 28×28 grayscale image, using a fixed-point (Q4.12-style) number format, and streams the results into an output buffer that a downstream module (likely ReLU/pooling) reads from via rd_addr/rd_data.

IMAGE_LOADING: Whenever img_wr_en is high, img_wr_data is latched into image_mem at img_wr_addr. This runs independently of the FSM and presumably is driven by whatever upstream module (camera/frame buffer) feeds pixels in.

MEMORY BLOCKS:

- image_mem[0:783] — the 28×28 = 784-pixel input image, loaded externally via img_wr_addr/img_wr_data/img_wr_en before computation starts.

- weight_mem[0:35] — 4 filters × 9 weights each (3×3 kernels) = 36 values, loaded from conv_weight.hex at simulation/elaboration start.

- bias_mem[0:3] — one bias per filter, loaded from conv_bias.hex.

- conv_out[0:2703] — output feature maps. With "valid" convolution (no padding) on a 28×28 image with a 3×3 kernel, each output map is 26×26 = 676 pixels; four filters give 4×676 = 2704 outputs, matching the array size.

module conv_full (

input wire clk,

input wire rst,

input wire img_load_done,

input wire [9:0] img_wr_addr,

input wire signed [15:0] img_wr_data,

input wire img_wr_en,

output reg done,

input wire [11:0] rd_addr,

output reg signed [15:0] rd_data

);

reg signed [15:0] image_mem [0:783];

reg signed [15:0] weight_mem [0:35];

reg signed [15:0] bias_mem [0:3];

initial begin

$readmemh("conv_weight.hex", weight_mem);

$readmemh("conv_bias.hex", bias_mem);

end

always @(posedge clk) begin

if (img_wr_en) begin

image_mem[img_wr_addr] <= img_wr_data;

end

end

reg [1:0] f; reg [4:0] i; reg [4:0] j;

reg signed [15:0] conv_out [0:2703];

reg signed [15:0] px [0:8]; reg signed [15:0] wt [0:8]; reg signed [15:0] bs;

reg signed [31:0] prod [0:8];

reg signed [31:0] s01, s23, s45, s67, s0123, s4567, sum_reg, result_reg;

reg [9:0] px_addr [0:8];

reg [1:0] f_reg;

reg [11:0] out_addr_ctr;

localparam WAIT_LOAD=0, CALC_ADDR=1, FETCH=2, MULT=3, SUMM1=4, SUMM2=5, SUMM3=6, WRITE=7;

reg [2:0] state;

integer n;

always @(posedge clk or posedge rst) begin

if (rst) begin

f<=0; i<=0; j<=0; done<=0; state<=WAIT_LOAD; out_addr_ctr<=0;

end else begin

case (state)

WAIT_LOAD: begin

if (img_load_done) state <= CALC_ADDR;

end

CALC_ADDR: begin

px_addr[0]<=(i+0)*28+j+0; px_addr[1]<=(i+0)*28+j+1; px_addr[2]<=(i+0)*28+j+2;

px_addr[3]<=(i+1)*28+j+0; px_addr[4]<=(i+1)*28+j+1; px_addr[5]<=(i+1)*28+j+2;

px_addr[6]<=(i+2)*28+j+0; px_addr[7]<=(i+2)*28+j+1; px_addr[8]<=(i+2)*28+j+2;

f_reg <= f;

state <= FETCH;

end

FETCH: begin

for (n=0;n<9;n=n+1) px[n] <= image_mem[px_addr[n]];

for (n=0;n<9;n=n+1) wt[n] <= weight_mem[f_reg*9+n];

bs <= bias_mem[f_reg];

state <= MULT;

end

MULT: begin

for (n=0;n<9;n=n+1) prod[n] <= px[n]*wt[n];

state <= SUMM1;

end

SUMM1: begin

s01<=prod[0]+prod[1]; s23<=prod[2]+prod[3];

s45<=prod[4]+prod[5]; s67<=prod[6]+prod[7];

state <= SUMM2;

end

SUMM2: begin

s0123<=s01+s23; s4567<=s45+s67;

state <= SUMM3;

end

SUMM3: begin

sum_reg <= s0123+s4567+prod[8]+bs*16'sd4096;

state <= WRITE;

end

WRITE: begin

result_reg = (sum_reg<0) ? 32'sd0 : sum_reg;

conv_out[out_addr_ctr] <= result_reg >>> 12;

out_addr_ctr <= out_addr_ctr + 1;

if (j==25) begin j<=0;

if (i==25) begin i<=0;

if (f==3) done<=1; else f<=f+1;

end else i<=i+1;end else j<=j+1;

state <= CALC_ADDR;

end

default: state <= WAIT_LOAD;

endcase

end

end

always @(posedge clk) rd_data <= conv_out[rd_addr];

endmodule

## maxpool.v

This is the max-pooling layer that sits right after conv_full.v in your pipeline; 2×2 max pooling with stride 2, reducing each 26×26 conv output map down to 13×13, across all 4 filters.

Data sizing

- Input: conv_rd_data pulled from conv_full's conv_out, which holds 4 filters × 26×26 = 2704 values.

- Output: pool_out[0:675] - 4 × 13×13 = 676 values (2×2/stride-2 pooling of 26×26 gives 13×13 per filter).

module maxpool (

input wire clk, input wire rst, input wire conv_done, output reg done,

output reg [11:0] conv_rd_addr, input wire signed [15:0] conv_rd_data,

input wire [9:0] rd_addr, output reg signed [15:0] rd_data

);

reg [1:0] f; reg [3:0] i; reg [3:0] j;

reg signed [15:0] pool_out [0:675];

reg [11:0] addr0_reg, addr1_reg, addr2_reg, addr3_reg;

reg [11:0] base_f;

reg signed [15:0] v0, v1, v2, v3;

reg signed [15:0] max01_reg, max23_reg, max_result_reg;

localparam CALC=0, A0=1,W0=2,L0=3, A1=4,W1=5,L1=6, A2=7,W2=8,L2=9,

A3=10,W3=11,L3=12, MAXCALC1=13, MAXCALC2=14, WR=15;

reg [3:0] state;

always @(posedge clk or posedge rst) begin

if (rst) begin f<=0;i<=0;j<=0;done<=0;state<=CALC;base_f<=0; end

else if (conv_done && !done) begin

case (state)

CALC: begin

addr0_reg<=base_f+(i*2+0)*26+(j*2+0); addr1_reg<=base_f+(i*2+0)*26+(j*2+1);

addr2_reg<=base_f+(i*2+1)*26+(j*2+0); addr3_reg<=base_f+(i*2+1)*26+(j*2+1);

state<=A0;

end

A0: begin conv_rd_addr<=addr0_reg; state<=W0; end

W0: state<=L0;

L0: begin v0<=conv_rd_data; state<=A1; end

A1: begin conv_rd_addr<=addr1_reg; state<=W1; end

W1: state<=L1;

L1: begin v1<=conv_rd_data; state<=A2; end

A2: begin conv_rd_addr<=addr2_reg; state<=W2; end

W2: state<=L2;

L2: begin v2<=conv_rd_data; state<=A3; end

A3: begin conv_rd_addr<=addr3_reg; state<=W3; end

W3: state<=L3;

L3: begin v3<=conv_rd_data; state<=MAXCALC1; end

MAXCALC1: begin

max01_reg<=(v0>v1)?v0:v1; max23_reg<=(v2>v3)?v2:v3;

state<=MAXCALC2;

end

MAXCALC2: begin max_result_reg<=(max01_reg>max23_reg)?max01_reg:max23_reg; state<=WR; end

WR: begin

pool_out[f*169+i*13+j] <= max_result_reg;

if (j==12) begin j<=0;

if (i==12) begin i<=0;

if (f==3) done<=1; else begin f<=f+1; base_f<=base_f+676; state<=CALC; end

end else begin i<=i+1; state<=CALC; end

end else begin j<=j+1; state<=CALC; end

end

default: state<=CALC;

endcase

end

end

always @(posedge clk) rd_data <= pool_out[rd_addr];

endmodule

## fc_layer.v

The final layer of the pipeline taking the flattened 676-value pooled feature map (4×13×13) and projecting it down to 10 class scores via fc_out0 – fc_out9, feeding the argmax module for the final digit-rank prediction.

module fc_layer (

input wire clk, input wire rst, input wire pool_done, output reg done,

output reg [9:0] pool_rd_addr, input wire signed [15:0] pool_rd_data,

output reg signed [31:0] fc_out0, fc_out1, fc_out2, fc_out3, fc_out4,

output reg signed [31:0] fc_out5, fc_out6, fc_out7, fc_out8, fc_out9

);

reg signed [15:0] fc_weight_mem [0:6759];

reg signed [15:0] fc_bias_mem [0:9];

initial begin

$readmemh("fc_weight.hex", fc_weight_mem);

$readmemh("fc_bias.hex", fc_bias_mem);

end

reg [3:0] out_idx; reg [9:0] k; reg [12:0] base_addr, weight_addr;

reg signed [15:0] weight_val_reg;

reg signed [31:0] product_reg;

reg signed [47:0] acc, final_val;

localparam IDLE=0, SET_ADDR=1, WAIT_READ=2, READ_MEM=3, MULT=4, ADD=5, ADD_BIAS=6, WRITE=7;

reg [2:0] state;

wire signed [15:0] bias_val = fc_bias_mem[out_idx];

always @(posedge clk or posedge rst) begin

if (rst) begin

out_idx<=0; k<=0; acc<=0; done<=0; state<=IDLE;

pool_rd_addr<=0; base_addr<=0;

end else begin

case (state)

IDLE: if (pool_done) begin acc<=0; k<=0; base_addr<=0; pool_rd_addr<=0; state<=SET_ADDR; end

SET_ADDR: begin pool_rd_addr<=k; weight_addr<=base_addr+k; state<=WAIT_READ; end

WAIT_READ: state<=READ_MEM;

READ_MEM: begin weight_val_reg<=fc_weight_mem[weight_addr]; state<=MULT; end

MULT: begin product_reg<=pool_rd_data*weight_val_reg; state<=ADD; end

ADD: begin

acc<=acc+product_reg;

if (k==675) state<=ADD_BIAS; else begin k<=k+1; state<=SET_ADDR; end

end

ADD_BIAS: begin final_val<=acc+(bias_val*32'sd4096); state<=WRITE; end

WRITE: begin

case (out_idx)

0: fc_out0<=final_val; 1: fc_out1<=final_val; 2: fc_out2<=final_val;

3: fc_out3<=final_val; 4: fc_out4<=final_val; 5: fc_out5<=final_val;

6: fc_out6<=final_val; 7: fc_out7<=final_val; 8: fc_out8<=final_val;

9: fc_out9<=final_val;

endcase

if (out_idx==9) done<=1;

else begin

out_idx<=out_idx+1; base_addr<=base_addr+676;

acc<=0; k<=0; pool_rd_addr<=0; state<=SET_ADDR;

end

end

default: state<=IDLE;

endcase

end

end

endmodule

## argmax.v

This layer takes the 10 value vector from fc_layer.v and produces the final prediction.

Structure

- vals[0:9] — a local register array that's loaded once, in parallel, from the 10 individual fc_out* ports the moment fc_done is seen. This snapshot decouples the argmax scan from the FC layer's outputs, so it doesn't matter if fc_layer changes those registers again later (e.g., on a subsequent inference run) while this scan is still in progress.

- max_val/max_idx — running best value and its index.

- idx/cur_val — loop counter and the currently-fetched candidate value.

module argmax (

input wire clk, input wire rst, input wire fc_done,

input wire signed [31:0] fc_out0, fc_out1, fc_out2, fc_out3, fc_out4,

input wire signed [31:0] fc_out5, fc_out6, fc_out7, fc_out8, fc_out9,

output reg done, output reg [3:0] predicted_digit

);

reg signed [31:0] vals [0:9];

reg signed [31:0] max_val, cur_val;

reg [3:0] max_idx, idx;

localparam LOAD=0, READ=1, CMP=2, FINISH=3;

reg [1:0] state;

always @(posedge clk or posedge rst) begin

if (rst) begin done<=0; state<=LOAD; end

else case (state)

LOAD: if (fc_done) begin

vals[0]<=fc_out0; vals[1]<=fc_out1; vals[2]<=fc_out2; vals[3]<=fc_out3; vals[4]<=fc_out4;

vals[5]<=fc_out5; vals[6]<=fc_out6; vals[7]<=fc_out7; vals[8]<=fc_out8; vals[9]<=fc_out9;

max_val<=fc_out0; max_idx<=0; idx<=1; state<=READ;

end

READ: begin cur_val<=vals[idx]; state<=CMP; end

CMP: begin

if (cur_val>max_val) begin max_val<=cur_val; max_idx<=idx; end

if (idx==9) state<=FINISH; else begin idx<=idx+1; state<=READ; end

end

FINISH: begin predicted_digit<=max_idx; done<=1; end

default: state<=LOAD;

endcase

end

endmodule

## cnn_core.v

Top-level file to combine all the CNN Verilog files together.

module cnn_core (

input wire clk50,

input wire start,

input wire img_load_done,

input wire [9:0] img_wr_addr,

input wire signed [15:0] img_wr_data,

input wire img_wr_en,

output wire [3:0] predicted_digit,

output wire done

);

wire pll_locked;

wire clk;

my_pll u_pll (

.refclk (clk50),

.rst (1'b0),

.outclk_0 (clk),

.locked (pll_locked)

);

wire rst = start | ~pll_locked;

wire conv_done, pool_done, fc_done, argmax_done;

wire [11:0] conv_rd_addr;

wire signed [15:0] conv_rd_data;

wire [9:0] pool_rd_addr;

wire signed [15:0] pool_rd_data;wire signed [31:0] fc_out0, fc_out1, fc_out2, fc_out3, fc_out4;

wire signed [31:0] fc_out5, fc_out6, fc_out7, fc_out8, fc_out9;

conv_full u_conv (

.clk(clk), .rst(rst), .done(conv_done),

.img_load_done(img_load_done),

.img_wr_addr(img_wr_addr),

.img_wr_data(img_wr_data),

.img_wr_en(img_wr_en),

.rd_addr(conv_rd_addr),

.rd_data(conv_rd_data)

);

maxpool u_pool (

.clk(clk), .rst(rst), .conv_done(conv_done), .done(pool_done),

.conv_rd_addr(conv_rd_addr),

.conv_rd_data(conv_rd_data),

.rd_addr(pool_rd_addr), .rd_data(pool_rd_data)

);

fc_layer u_fc (

.clk(clk), .rst(rst), .pool_done(pool_done), .done(fc_done),

.pool_rd_addr(pool_rd_addr), .pool_rd_data(pool_rd_data),

.fc_out0(fc_out0), .fc_out1(fc_out1), .fc_out2(fc_out2), .fc_out3(fc_out3), .fc_out4(fc_out4),

.fc_out5(fc_out5), .fc_out6(fc_out6), .fc_out7(fc_out7), .fc_out8(fc_out8), .fc_out9(fc_out9)

);

argmax u_argmax (

.clk(clk), .rst(rst), .fc_done(fc_done),

.fc_out0(fc_out0), .fc_out1(fc_out1), .fc_out2(fc_out2), .fc_out3(fc_out3), .fc_out4(fc_out4),

.fc_out5(fc_out5), .fc_out6(fc_out6), .fc_out7(fc_out7), .fc_out8(fc_out8), .fc_out9(fc_out9),

.done(argmax_done), .predicted_digit(predicted_digit)

);

assign done = argmax_done;

endmodule

# Connecting the D8M to the FPGA

The D8M section consists of 3 main files that you should focus on, which is

- camera_capture.v which performs all the camera i2c, mipi i2c and HDMI feed functions

- downsample_28x28.v which down-samples all incoming frame to 28 by 28 to fit into the CNN

- capture_snapshot.v which captures the screenshot from the down-sample and feeds to the CNN

## camera_capture.v

This is the top-level camera capture and display module — it ties together the MIPI camera → HDMI passthrough display path, plus a parallel tap that downsamples the live video into a 28×28 snapshot buffer for the CNN (conv_full's img_wr_addr/img_wr_data interface from earlier). It's the missing piece connecting your D8M camera to the CNN pipeline we discussed.

The VGA-domain RGB stream is fed into the downsample.v (explained later) through the RAW2RGB function which converts the RD_DATA from FRAM.  This VGA Domain is also fed into the HDMI signal so we can see the picture.

Pathway for LIVE HDMI FEED

MIPI camera → ON_CHIP_FRAM (dual-clock FIFO/frame buffer) → RAW2RGB_J (Bayer demosaic) → VGA_Controller (timing) → HDMI_TX_AD7513 (HDMI transmitter)

Pathway for CNN Snapshot

VGA-domain RGB stream → downsample_28x28 → capture_snapshot → hps_rd_addr/hps_rd_data (feeds conv_full's image loader)

module camera_capture (

input wire clk50,

input wire clk2_50,

input wire rst_n,

input wire capture_trigger_in,

inout wire HDMI_I2C_SCL, inout wire HDMI_I2C_SDA,

inout wire HDMI_I2S, inout wire HDMI_LRCLK,

inout wire HDMI_MCLK, inout wire HDMI_SCLK,

output wire HDMI_TX_CLK, output wire HDMI_TX_DE,

output wire [23:0] HDMI_TX_D, output wire HDMI_TX_HS,

input wire HDMI_TX_INT, output wire HDMI_TX_VS,

inout wire CAMERA_I2C_SCL, inout wire CAMERA_I2C_SDA,

output wire CAMERA_PWDN_n, output wire MIPI_CS_n,

inout wire MIPI_I2C_SCL, inout wire MIPI_I2C_SDA,

output wire MIPI_MCLK, input wire MIPI_PIXEL_CLK,

input wire [9:0] MIPI_PIXEL_D,

input wire MIPI_PIXEL_HS, input wire MIPI_PIXEL_VS,

output wire MIPI_REFCLK, output wire MIPI_RESET_n,

output wire dbg_mipi_release, dbg_camera_release, dbg_hdmi_ready, dbg_pll_ok,

input wire [9:0] hps_rd_addr,

output wire [15:0] hps_rd_data,

output wire snapshot_done

);

wire RESET_N, RESET_N_DELAY;

wire MIPI_BRIDGE_RELEASE, CAMERA_MIPI_RELAESE;

wire AUD_CTRL_CLK, PLL_TEST_OK, VGA_CLK;

wire [9:0] RD_DATA;

wire [19:0] WR_ADDR, RD_ADDR;

wire [7:0] RED, GREEN, BLUE;

wire VGA_HS, VGA_VS, READ_Request, HDMI_READY;

wire [10:0] cur_x, cur_y;

wire HDMI_I2S_int;

assign CAMERA_PWDN_n = 1'b1;

assign MIPI_CS_n = 1'b0;

assign MIPI_RESET_n = RESET_N;

RESET_DELAY_HDMI dl (

.RESET_N (rst_n), .CLK (clk50),

.READY0 (RESET_N), .READY1 (RESET_N_DELAY)

);

MIPI_BRIDGE_CAMERA_Config cfin (

.RESET_N (RESET_N_DELAY), .CLK_50 (clk50),

.MIPI_I2C_SCL (MIPI_I2C_SCL), .MIPI_I2C_SDA (MIPI_I2C_SDA),

.MIPI_I2C_RELEASE (MIPI_BRIDGE_RELEASE),

.CAMERA_I2C_SCL (CAMERA_I2C_SCL), .CAMERA_I2C_SDA (CAMERA_I2C_SDA),

.CAMERA_I2C_RELAESE (CAMERA_MIPI_RELAESE)

);

AUDIO_PLL pll1 (.refclk(clk50), .rst(1'b0), .outclk_0(AUD_CTRL_CLK), .locked(PLL_TEST_OK));

VIDEO_PLL pll2 (.refclk(clk2_50), .rst(1'b0), .outclk_0(MIPI_REFCLK), .outclk_1(VGA_CLK));

ON_CHIP_FRAM fra (

.W_CLK(MIPI_PIXEL_CLK), .W_DE(MIPI_PIXEL_HS & MIPI_PIXEL_VS),

.W_DATA(MIPI_PIXEL_D[9:0]), .W_CLR(MIPI_PIXEL_VS),

.R_CLK(VGA_CLK), .R_DATA(RD_DATA), .R_CLR(VGA_VS), .R_DE(READ_Request),

.WR_ADDR(WR_ADDR), .RD_ADDR(RD_ADDR)

);

RAW2RGB_J u4 (

.RST(VGA_VS), .CCD_PIXCLK(VGA_CLK), .mCCD_DATA(RD_DATA[9:0]),

.VGA_CLK(VGA_CLK), .READ_Request(READ_Request),

.VGA_VS(VGA_VS), .VGA_HS(VGA_HS),

.oRed(RED), .oGreen(GREEN), .oBlue(BLUE), .oDVAL()

);

VGA_Controller u1 (

.iRed(10'd0), .iGreen(10'd0), .iBlue(10'd0),

.oCurrent_X(cur_x), .oCurrent_Y(cur_y), .oAddress(),

.oRequest(READ_Request),

.oVGA_R(), .oVGA_G(), .oVGA_B(),

.oVGA_HS(VGA_HS), .oVGA_VS(VGA_VS),

.oVGA_SYNC(), .oVGA_BLANK(), .oVGA_CLOCK(),

.iCLK(VGA_CLK), .iRST_N(1'b1)

);

HDMI_TX_AD7513 hdmi (

.RESET_N(RESET_N), .CLK_50(clk50),

.AUD_CTRL_CLK(AUD_CTRL_CLK), .PLL_TEST_OK(PLL_TEST_OK),

.HDMI_I2C_SCL(HDMI_I2C_SCL), .HDMI_I2C_SDA(HDMI_I2C_SDA),

.HDMI_I2S(HDMI_I2S_int), .HDMI_LRCLK(HDMI_LRCLK),

.HDMI_MCLK(HDMI_MCLK), .HDMI_SCLK(HDMI_SCLK),

.HDMI_TX_INT(HDMI_TX_INT), .READY(HDMI_READY)

);

assign HDMI_TX_CLK = VGA_CLK;

assign HDMI_TX_D = READ_Request ? {RED, GREEN, BLUE} : 24'd0;

assign HDMI_TX_DE = READ_Request;

assign HDMI_TX_HS = VGA_HS;

assign HDMI_TX_VS = VGA_VS;

assign HDMI_I2S = 1'b0;

assign dbg_mipi_release = MIPI_BRIDGE_RELEASE;

assign dbg_camera_release = CAMERA_MIPI_RELAESE;

assign dbg_hdmi_ready = HDMI_READY;

assign dbg_pll_ok = PLL_TEST_OK;

wire [9:0] snap_src_addr;

wire signed [15:0] snap_src_data;

wire frame_ready;

downsample_28x28 u_downsample (

.vga_clk(VGA_CLK), .vga_vs(VGA_VS),.cur_x(cur_x), .cur_y(cur_y),

.pixel_valid(READ_Request),

.red(RED), .green(GREEN), .blue(BLUE),

.frame_ready(frame_ready),

.rd_addr(snap_src_addr), .rd_data(snap_src_data)

);

// Synchronize the HPS-driven trigger (50 MHz domain) into the VGA_CLK (25 MHz) domain

reg [2:0] trig_sync;

always @(posedge VGA_CLK) trig_sync <= {trig_sync[1:0], capture_trigger_in};

wire capture_trigger_vga = trig_sync[1] & ~trig_sync[2];

wire signed [15:0] snap_data_signed;

capture_snapshot u_snapshot (

.clk(VGA_CLK), .capture_trigger(capture_trigger_vga),

.frame_ready(frame_ready),

.src_rd_addr(snap_src_addr), .src_rd_data(snap_src_data),

.rd_addr(hps_rd_addr), .rd_data(snap_data_signed),

.snapshot_done(snapshot_done)

);

assign hps_rd_data = snap_data_signed;

endmodule

## downsample_28x28.v

downsample_28x28.v removes all the colours to grayscale and downsamples the signal down to 28x28 (simple!) when the frame is ready, reg frame_ready is triggered.

module downsample_28x28 (

input wire vga_clk,

input wire vga_vs,

input wire [10:0] cur_x,

input wire [10:0] cur_y,

input wire pixel_valid,

input wire [7:0] red, green, blue,

output reg frame_ready,

input wire [9:0] rd_addr,

output reg signed [15:0] rd_data

);

reg signed [15:0] frame_out [0:783];

reg [17:0] accum [0:783];

reg vs_prev;

wire frame_start = vga_vs & ~vs_prev;

wire in_crop = (cur_x >= 82) && (cur_x < 558) && (cur_y >= 2) && (cur_y < 478);

wire [8:0] crop_x = cur_x - 82;

wire [8:0] crop_y = cur_y - 2;

wire [4:0] cell_x = crop_x / 17;

wire [4:0] cell_y = crop_y / 17;

wire [9:0] cell_idx = cell_y * 28 + cell_x;

wire [9:0] gray = (255*3) - (red + green + blue); // invert: white bg -> dark, dark digit -> bright

reg committing;

reg [9:0] commit_idx;

always @(posedge vga_clk) begin

vs_prev <= vga_vs;

if (frame_start && !committing) begin

committing <= 1;

commit_idx <= 0;

end else if (committing) begin

frame_out[commit_idx] <= (accum[commit_idx] * 227) >> 12; // approximates divide-by-289

accum[commit_idx] <= 0;

if (commit_idx == 783) begin

committing <= 0;

frame_ready <= 1;

end else begin

commit_idx <= commit_idx + 1;

frame_ready <= 0;

end

end else begin

frame_ready <= 0;

if (pixel_valid && in_crop)

accum[cell_idx] <= accum[cell_idx] + gray;

end

end

always @(posedge vga_clk) rd_data <= frame_out[rd_addr];

endmodule

## capture_snapshot.v

When frame_ready is triggered,  it copies from the live frame buffer into its own private snapshot[] array only once per trigger. This is then fed into the CNN.

module capture_snapshot (

input wire clk,

input wire capture_trigger,

input wire frame_ready,

output reg [9:0] src_rd_addr,

input wire signed [15:0] src_rd_data,

input wire [9:0] rd_addr,

output reg signed [15:0] rd_data,

output reg snapshot_done

);

reg signed [15:0] snapshot [0:783];

reg [9:0] copy_idx;

localparam IDLE=0, WAIT_FRAME=1, SET_ADDR=2, WAIT_READ=3, WRITE=4;

reg [2:0] state;

always @(posedge clk) begin

case (state)

IDLE: begin

if (capture_trigger) begin

snapshot_done <= 0;

state <= WAIT_FRAME;

end

end

WAIT_FRAME: if (frame_ready) begin

copy_idx <= 0;

state <= SET_ADDR;

end

SET_ADDR: begin

src_rd_addr <= copy_idx;

state <= WAIT_READ;

end

WAIT_READ: state <= WRITE;

WRITE: begin

snapshot[copy_idx] <= src_rd_data;

if (copy_idx == 783) begin

snapshot_done <= 1;

state <= IDLE;

end else begin

copy_idx <= copy_idx + 1;

state <= SET_ADDR;

end

end

default: state <= IDLE;

endcase

end

always @(posedge clk) rd_data <= snapshot[rd_addr];

endmodule

# CNN Inference through HPS BareMetal

Program the FPGA and load up the given Baremetal Application and run through the program to get the result as below.

# ACTIVITIES: WHAT’S NEXT?

You may have noticed that your mini-project needs your to scan and infer multiple cards. As such you need to modify the CNN for the following functions

- SWITCH FROM MNIST TO PLAYING CARDS OF THE 13 MEMBERS OF EACH SUIT

- READ MULTIPLE CARDS ON ONE INFERENCE (AT LEAST 2 CARDS BY MILESTONE 1)

- NOTE THAT THIS STRUCTURE IS STATIC AND USES AROUND 16000 CYCLES TO COMPLETE ONE INFERENCE. ADD PIPELINING

- ADD DECISION MAKING ON THE HPS SIDE TO MAKE YOUR CHOICES FOR YOUR FINAL MINIPROJECT RESULT.